Memorandum

To:	Andres Fergadiotti, SCE

From:	Yizhi Yang, Justin DeBlois, TRC

Date:	February 27, 2026

Re: 	Technical memo for DEER Nonresidential Prototype Scorecards

# DEER Prototype Scorecard Automated Generation

## Purpose and Scope

The CPUC established the DEER nonresidential prototype building energy models in a new EnergyPlus modeling framework in 2023. While this effort completely revamped the energy models, no documentation of the model development process or the model inputs and assumptions exists. The CPUC, SCE and other stakeholders noted that this lack of documentation has made measure development and measure review more difficult. In addition, while several stakeholders noted differences in EUI and savings between the EnergyPlus prototypes, the previous generation of DOE-2 prototypes and other prototypes such as the CalBEM prototype models, there is no readily available reference documentation on the DEER prototypes. Establishing this documentation is seen as a key step in evaluating the DEER prototype models and comparing against recent efforts to characterize the building stock such as the CalBEM prototypes.

The purpose of this work is to document the DEER nonresidential prototype inputs by developing an automated process to create prototype scorecards. This work was requested by the CPUC and measure developers to support measure review and development and is timed to align with PY28 and the release of the CalBEM scorecards. Parallel release of the DEER scorecards will enable easy comparison between the two sets of prototypes. Developing and releasing an automated process for generating scorecards will allow stakeholders to update the scorecards in the future as the DEER repository continues to evolve with new measures, bug fixes and improvements.

The scorecard project includes the 26 nonresidential DEER prototype models. The project team ran each building type for two vintages, 16 climate zones and five main HVAC systems. The prototype version is the DEER repository as of 2/19/2026. The scorecard format and the data fields included in the scorecards are based on feedback from the eTRM Modeled Measures SME meeting held on 12/9/2025 and sample CalBEM prototypes provided on 12/9/2025. The project team made additional adjustments to the format and data fields to improve clarity and automation.

## Methodology

### Overview of Scorecard Automated Generation

The team proposes a workflow to enable automated generation of DEER prototype scorecards, as shown in Figure 1. The workflow can be divided into four main steps:

- DEER prototype model generation and simulation, which produces EnergyPlus (EP) input and output files (e.g., idf, eio, html)

- Data extraction automation using Python to obtain values

- Post-processing with rule-based methods to transform and standardize the extracted data such as zone type categorization

- Scorecard generation, in which raw data is organized, additional values are computed using predefined equations in the spreadsheet, and the finalized results are populated across Excel tabs to produce the complete scorecard.

Figure 1. Workflow of scorecard automated generation

### DEER Prototype Model Generation and Simulation

The DEER nonresidential prototype models use the Modelkit framework and draw from the DEER repository as of 2/19/2026. A Python workflow then automatically extracts and sorts the generated models and runs EnergyPlus (v22.2) to produce the required outputs. For example, the .eio file includes both standard and optional reports that can be easily imported into spreadsheet programs for further analysis. The weather file dictionary is hard-coded in the Python script to link the corresponding weather file to each climate zone (CZ) number for EnergyPlus simulations, as shown in Figure 2.

Figure 2. Hard-coded weather file dictionary in the Python script for EP simulation

### Data Extraction Automation

Python-based automated scripts extract the data. A separate extraction function is used for each building attribute, such as zone summary, and HVAC system, to account for differences in building energy model data formats. The team organizes these extraction functions by data category and stores them in a Python script library to support modular data extraction and easier code management. These functions automatically parse EnergyPlus input (.idf) and output (.htm and .eio) files and export raw tables for downstream processing. Prototype names are hard coded in the Python script by linking each prototype to its abbreviation, as shown in Figure 3.

Figure 3. Hard-coded prototype full name dictionary in the Python script in raw data extraction

### Data Post-processing

The team groups internal EP model zones into defined zone type categories based on the original DEER prototype root files. The team uses natural language zone type names to improve readability. Python rule-based post-processing scripts automate this classification and aggregation step. After the team assigns zone types, the team aggregates key parameters by zone type. These parameters include lighting power density and plug load density. This supports consistent comparisons across zone types. The team also uses schedule identifier labels to group zone-level schedules. This makes schedule types easier to interpret. Some buildings include HVAC systems beyond the main configuration. For example, for the secondary school with HVAC name cAVVG, the boiler and chiller system is the main heating and cooling system, but it also has a furnace and direct expansion air conditioning systems serving the computer room and dining room. The team adds these systems to ensure the HVAC system information is complete. Dictionary-based rules identify and include these supplemental HVAC systems automatically.

All classifications and supplements rely on a dictionary that the team creates from the original DEER prototype root files. The dictionary uses a .json file format. The workflow uses the dictionary to identify additional HVAC systems, assign zone categories, and label the schedule identifier for each zone in the EP model. The Python workflow automatically reads and applies this .json dictionary during post-processing. The team can also edit the .json file to customize zone type names.

### Spreadsheet Scorecard Generation

The team creates a spreadsheet template with predefined formulas but without any DEER prototype data in presenting Tab. The template includes a Geometry Summary tab to store the prototype shape image, aspect ratio, number of floors, floor-to-floor height, and floor-to-ceiling height. It also includes tabs for storing data extracted from EnergyPlus and tabs for presenting the scorecard results.

Python scripts populate the template by mapping the processed datasets into the appropriate tabs. The extracted data populates the designated tabs. The team then hides these tabs and uses them as source data for the scorecard presentation tabs. However, the tabs can be unhidden for review or exploratory purposes as needed. The workflow automatically generates the final scorecard files for each prototype without manual copy/paste.

## Instructions for Using the Script

A Python-based script execution workflow was developed to automate the generation of DEER nonresidential prototype scorecards. The process is organized into three sequential runs:

- Prepare & Simulate, which sorts inputs and executes EP models. Run 1 implements Step 1 of the methodology.

- Extract & Process, which extracts relevant results using a function library, applies rule-based intermediate processing, and compiles outputs into structured datasets. Run 2 implements Steps 2 and 3 of the methodology.

- Generate Scorecard, which converts the processed data into Excel tabs and populates a predefined scorecard template with built-in equations. Run 3 implements Step 4 of the methodology.

These three stages form an end-to-end automation pipeline that streamlines simulation and post-processing and ensures consistent, reproducible scorecard production across prototypes.

The user will start in the DEER-Prototypes-EnergyPlus-Working/ScorecardAutomation folder. After generating the DEER prototypes using the ModelKit workflow, the user will go to the ScorecardAutomation folder and run three Python scripts: Run1_Pre_Data_Extraction.py, Run2_Data_Extraction_Processing.py, and Run3_Scorecard_Generation.py. Before running the scripts, the user must update the required paths, including the DEER prototype folder path, the EP installation path, and the weather file path. Then the user runs the scripts in order: Run1, Run2, and Run3. Run1 will take the longest because it includes the EnergyPlus simulations. The supporting modules locate in DEER-Prototypes-EnergyPlus-Working /ScorecardAutomation/src. Figure 4 shows how the run scripts relate to the supporting scripts and templates. The expected outputs for each step are described in the following subsections.

Figure 4. Python-based script execution workflow

### Environment Setup Requirements

Before running the three scripts in order Run1_Pre_Data_Extraction.py, Run2_Data_Extraction_Processing.py, Run3_Scorecard_Generation.py. Two tables are provided for users to review the suggested system and software requirements and to complete the pre-run checklist. Table 1 summarizes the recommended setup and Table 2 presents the pre-run checklist.

Table 1. Recommendation for system and software requirements

| Name | Recommendation |
| --- | --- |
| Operating System | Windows 11 (recommended) |
| Software | Python 3 (recommended 3.10+) EnergyPlus installed (recommended EnergyPlusV22-2-0 given DEER prototype version) Microsoft Excel Desktop |

Table 2. Pre-run checklist

| Number | Checklist |
| --- | --- |
| 1 | Python runs and points to intended environment |
| 2 | EP folder exists and contains energyplus.exe |
| 3 | Weather folder contains required .epw files |
| 4 | ScorecardAutomation/src exists and contains scripts 0–6 |
| 5 | Run1/Run2/Run3 “ONLY EDIT PATHS HERE” blocks updated |
| 6 | Scorecard template file path is correct and Excel desktop works |

### Preparation and Simulation

After running Pre_Data_Extraction.py, the workflow executes two scripts in sequence: 0_EP_Input_Sort.py followed by 1_EP_Run.py. 0_EP_Input_Sort.py collects the EP input files from the Modelkit runs directories and reorganizes them into a clean folder structure under DEER-Prototypes-EnergyPlus-Working\ScorecardAutomation\EP_File by building type, vintage category, and California Energy Commission (CEC) climate zone. DEER-Prototypes-EnergyPlus-Working is the folder used for DEER prototype generation, and ScorecardAutomation is a subdirectory within the DEER-Prototypes-EnergyPlus-Working repository. The automation scripts use the ScorecardAutomation folder as the working module and reference shared repository resources (e.g., weather/, commercial measures/) via paths relative to the repository root. Next, 1_EP_Run.py iterates through the organized IDF folders, selects the appropriate EPW weather file based on the climate-zone folder name (CZ01–CZ16) and the weather-file dictionary in 1_EP_Run.py. It then programmatically updates each IDF to enforce the Inch-Pound table style and runs EP from the command line. The script saves outputs back to the same folder with a consistent prefix for later extraction and scorecard population. The resulting file and folder structure that organizes IDF files by climate zone, vintage, and prototype is shown in Figure 5.

| a. Folder path: EP_File | b. Folder path: EP_File/Asm |
| --- | --- |
| c. Folder path: EP_File/Asm/Existing | d. Folder path: EP_File/Asm/Existing/CZ01 |

Figure 5. Folder structure for the assembly prototype after running the pre-data processing scripts

### Data Extraction and Processing

After running Data_Extraction_Processing.py, the workflow runs 2_Main.py, 3_Intermediate_Process.py, and 4_Merge_Result.py in sequence, and the workflow creates one final compiled data folder (Compiled_Data) and two intermediate data folders (Extracted_Data_Raw and Extracted_Data_Intermediate_Processed) under the EP_Files folder. The source-file mapping used for building attributes is documented in Appendix A Table A. 1, and the unit conversion rules applied during data extraction and processing are documented in Appendix A Table A. 2.

The script 2_Main.py acts as the primary extraction executor. It imports parsing functions from Extraction_Lib.py, walks through the organized EP_Files folder by building type and vintage, and reads the EP result files (.idf, .htm, and .eio) to extract key tables such as building metadata, window dimensions and sill height, shading geometry, zone summary, envelope, infiltration, water heater, OA controller, DHW peak flow, annual end uses, end-use breakdown, HVAC, fan, refrigeration, thermostat setpoints, schedules, and gas equipment.

During this extraction stage, Extraction_Lib.py performs required unit conversions to standardize values to Inch-Pound (IP) units where EP input (.idf) and output (.eio) remain in SI units (for example, converting geometry from meters to feet, airflow from m³/s to cfm, infiltration from m³/s·m² to cfm/ft², equipment intensity from W/m² to Btu/h-ft², fan pressure from Pa to inches water column, envelope U-values from W/m²·K to Btu/h-ft²·°F and hot water volume flow rate from m³/s to gal/hr). It then writes them into a structured multi-section CSV in Extracted_Data_Raw. The generated folder and datasets are shown in Figure 6.

Figure 6. Folder of Extracted_Data_Raw

Next, the script 3_Intermediate_Process.py applies rule-based post-processing to these raw extracted tables. It generates and uses a zone-type and HVAC-identifier mapping from the prototype root.pxt files, applies aggregation rules (for example, area-weighted summaries using zone multipliers), and standardizes or enriches tables (such as adding HVAC and schedule identifiers to summary tables) to produce cleaner intermediate CSVs in Extracted_Data_Intermediate_Processed. This step also performs key unit conversions for downstream aggregation and scorecard calculations, including converting thermostat setpoints and temperature schedules from °C to °F and converting lighting and plug intensities from Btu/h to Watts where required. The generated folder and datasets are shown in Figure 7.

Figure 7. Folder of Extracted_Data_Intermediate_Processed.

Next, the script 4_Merge_Result.py compiles the processed outputs by integrating results across the five HVAC configurations and two vintages into a consolidated dataset for each prototype. It merges and de-duplicates the intermediate CSV sections, appends vintage and HVAC configuration tags, and writes a single compiled output file named by prototype to the Compiled_Data folder.The generated folder and datasets are shown in Figure 8.

Figure 8. Folder of Compiled_Data

### Automated Scorecard Generation

After running the Scorecard_Generation.py, 0_Raw2Excel.py converts each compiled .csv file in EP_Files\Compiled_Data into an Excel workbook saved under Tab_from_Raw, as shown in Figure 9. Each table section in the CSV becomes a separate worksheet with a Raw_ prefix and a corresponding sheet name, as shown in Figure 10.

Figure 9. Folder of excel workbook that extract data from run 2

Figure 10. Sheet name inside each Excel workbook

Next, the script 1_Tab_Import.py opens the scorecard template (DEER_PrototypeName_ScoreCard.xlsx) which includes predefined formulas, and the corresponding raw tab workbook. It copies each raw sheet into the matching template tab and hides the Raw_ tabs. It removes trailing placeholder rows in selected tabs. It generates the Schedule figures and keeps the charts linked to the Schedules tab. Finally, it saves a finalized scorecard workbook for each prototype, as shown in Figure 11.

Figure 11. Folder of final prototype scorecards

Appendix A

Table A. 1. Source file mapping for building attributes in Run 2

| Building Attribute | Source File |
| --- | --- |
| Building Type | DEER prototype name |
| Building Category | DEER prototype name |
| Roof Type | idf |
| Window-Wall Ratio | html |
| Window Dimensions | eio |
| Window Sill Height | idf |
| Shading Geometry | idf |
| Run Begin Day | idf |
| Special Days | idf |
| Zone Area / Volume | html |
| Zone Multipliers | html |
| Lighting Density | html |
| Plug Load Density | html |
| People Density | html |
| Outdoor Air (per area/person) | idf |
| Zone Type Mapping | pxt |
| HVAC Identifier | pxt |
| Construction Layers | idf |
| Roof/Wall U-Value | html |
| Window U-Value / SHGC / VT | idf |
| HVAC Parameters | idf |
| Fan Parameters | idf + html |
| Gas Equipment | idf |
| Refrigeration System | idf |
| Thermostat Setpoints | idf |
| Water Heater Properties | idf + html |
| DHW Peak Flow by Zone | idf |
| Total End Uses | html |
| End Use Breakdown | html |
| Annual Electricity / Gas | html |
| Schedule Profiles | eio + idf |

Table A. 2. Unit conversion of building attributes during Run 2

| Building Attribute | Start unit | Converted unit (output after Run 2) | Processed scripts |
| --- | --- | --- | --- |
| Window width / height | m | ft | Extraction_Lib.py (during 2_Main.py) |
| Window sill height | m | ft |  |
| OA flow per person | m³/s-person | cfm/person |  |
| OA flow per floor area | m³/s·m² | cfm/ft² |  |
| Infiltration rate | m³/s·m² | cfm/ft² |  |
| Gas equipment power density | W/m² | Btu/h-ft² |  |
| Fan static pressure | Pa | in. w.c. |  |
| Window U-value | W/m²·K | Btu/h-ft²-°F |  |
| DHW peak flow | m³/s | gal/hr |  |
| Thermostat setpoints | °C | °F | 3_Intermediate_Process.py |
| Temperature setpoint schedules | °C | °F |  |
| Lighting / plug intensity | Btu/h-ft² | W/ft² |  |